Définitions des champs

Scapy

# Protocole Ethernet

| Champ | Champ Scapy | Valeur(s) possible(s) |
| --- | --- | --- |
| Adresse MAC source | src | Adresse MAC, format 00:00:00:00:00:00, string |
| Adresse MAC destination | dst | Adresse MAC, format 00:00:00:00:00:00, string |
| Protocole encapsulé | type | Hexa, format 0x800 Valeurs possibles courantes : IP : 0x0800 ARP : 0x806 |

Couche Ethernet, par packet.show() :

###[ Ethernet ]###

dst       = 98:fc:11:3d:30:b0

src       = b4:75:0e:b1:92:a1

type     = 0x800

# Protocole ARP

| Champ | Champ Scapy | Valeur(s) possible(s) |
| --- | --- | --- |
| Type du paquet (opcode) | op | Request : who-is (hexa/int = 1) Reply : is-at (hexa/int = 2) |
| Adresse MAC source | hwsrc | Adresse MAC, format 00:00:00:00:00:00, string |
| Adresse IP source | psrc | Adresse IP, format 255.255.255.255, string |
| Adresse MAC destination | hwdst | Adresse MAC, format 00:00:00:00:00:00, string |
| Adresse IP destination | pdst | Adresse IP, format 255.255.255.255, string |

Couche ARP, par packet.show() :

###[ ARP ]###

hwtype   = 0x1

ptype     = 0x800

hwlen     = 6

plen      = 4

op        = who-has

hwsrc     = 98:fc:11:3d:30:b0

psrc      = 192.168.1.1

hwdst     = 00:00:00:00:00:00

pdst      = 192.168.1.232

# Protocole IP

| Champ | Champ Scapy | Valeur(s) possible(s) |
| --- | --- | --- |
| Protocole encapsulé | proto | Request : who-is (hexa/int = 1) Reply : is-at (hexa/int = 2) |
| Adresse IP source | src | Adresse IP, format 255.255.255.255, string |
| Adresse IP destination | dst | Adresse IP, format 255.255.255.255, string |

Couche IP, par packet.show() :

###[ IP ]###

version   = 4L

ihl       = 5L

tos       = 0x0

len       = 84

id        = 994

flags     =

frag      = 0L

ttl       = 63

proto     = icmp

chksum    = 0x91c

src       = 192.168.1.242

dst       = 172.16.0.1

# Protocole ICMP

| Champ | Champ Scapy | Valeur(s) possible(s) |
| --- | --- | --- |
| Type du paquet | type | Echo-Request (8), Echo-Reply(0) |
| Code du paquet | code | Hexa (dépend du type) |

Couche ICMP, par packet.show() :

###[ ICMP ]###

type      = echo-request

code      = 0

chksum    = 0x15a0

id        = 0x57b3

seq       = 0x0

# Protocole UDP

| Champ | Champ Scapy | Valeur(s) possible(s) |
| --- | --- | --- |
| Port source | sport | Int (53,67,68, etc.) |
| Port destination | dport | Int (53,67,68, etc.) |

###[ UDP in ICMP ]###

sport     = domain

dport     = 27998

len       = 59

chksum    = 0xfd01

# Protocole DNS

| Champ | Champ Scapy | Valeur(s) possible(s) |
| --- | --- | --- |
| Flag Query-Response | qr | A one bit field that specifies whether this message is a query (0), or a response (1). |
| Type de requête | opcode | A four bit field that specifies kind of query in this message. This value is set by the originator of a query and copied into the response. The values are: - 0 a standard query (QUERY) - 1 an inverse query (IQUERY) - 2 a server status request (STATUS) |
| Autorité (Authoritive Answer) | aa | Authoritative Answer - this bit is valid in responses, and specifies that the responding name server is an authority for the domain name in question section. - 0 = No authority - 1 = Authority |
| Récursion désirée | rd | Recursion Desired - this bit may be set in a query and is copied into the response. If RD is set, it directs the name server to pursue the query recursively. Recursive query support is optional. |
| Récursion disponible | ra | Recursion Available - this is set or cleared in a response, and denotes whether recursive query support is available in the name server. |
| Zéro | z | Reserved for future use. Must be zero in all queries and responses. |
| Code réponse | RCODE | Response code - this 4 bit field is set as part of responses. The values have the following interpretation: - 0 No error condition - 1 Format error - The name server was unable to interpret the query. - 2 Server failure - The name server was unable to process this query due to a problem with the name server. - 3 Name Error - Meaningful only for responses from an authoritative name server, this code signifies that the domain name referenced in the query does not exist. |
| Nombre de questions | QDCOUNT | an unsigned 16 bit integer specifying the number of entries in the question section. |
| Nombre de réponses | ANCOUNT | an unsigned 16 bit integer specifying the number of resource records in the answer section. |
| Nombre d’entrées pour le nom des serveurs | NSCOUNT | an unsigned 16 bit integer specifying the number of name server resource records in the authority records section. |
| Nombre d’entrées pour le nombre de ressources additionnelles | ARCOUNT | an unsigned 16 bit integer specifying the number of resource records in the additional records section. |

## DNS Response:

###[ DNS ]###

id        = 48265

qr        = 1L

opcode    = QUERY

aa        = 0L

tc        = 0L

rd        = 1L

ra        = 1L

z         = 0L

ad        = 0L

cd        = 0L

rcode     = ok

qdcount   = 1

ancount   = 1

nscount   = 0

arcount   = 0

\qd        \

|###[ DNS Question Record ]###

Dans cette réponse, il y a d’abord la ‘question’ à laquelle on envoie une réponse (du paquet DNS query), et ensuite, la réponse envoyée par le serveur DNS

|  qname     = 'www.linuxmint.com.'

|  qtype     = A

|  qclass    = IN

\an        \

|###[ DNS Resource Record ]###

|  rrname    = 'www.linuxmint.com.'

|  type      = A

|  rclass    = IN

|  ttl       = 2602

|  rdlen     = 4

|  rdata     = '213.175.215.218'

ns        = None

ar        = None

Source : http://tools.ietf.org/html/rfc1035, RFC 1035 sur Domain Names, Implementations and Specifications